# Total Marks:	04	 Obtained Marks:

# Feasibility Study

Advanced Lane Detection System

Submitted To:

Mr. Muhammad Azhar

Submitted By:

Jahanzaib Ibrahim Khan ( 23108116 ) Mazher Ali Shah ( 23108121 )

# 1. Project Overview

This report documents the successful completion of the Advanced Road Lane Detection System as a university-level project in the domain of Computer Vision, Image Processing, and Autonomous Driving. Originally proposed as a feasibility study, the project has been fully implemented based on the Udacity Self-Driving Car Engineer Nanodegree curriculum. A complete software pipeline for identifying road lane boundaries in real-time video streams has been built, tested, and demonstrated.

## 1.1 Project Title

Advanced Lane Detection of the Road using Computer Vision Techniques

## 1.2 Project Description

The project applies a series of classical image processing and computer vision techniques to detect and visualize road lane lines from dashcam video footage. The implemented system successfully augments video output with the detected lane boundaries, road radius of curvature, and vehicle center offset — key metrics for autonomous driving systems.

## 1.3 Core Pipeline Steps — Implemented

- Camera calibration using chessboard images to compute distortion coefficients ✔ DONE

- Distortion correction applied to raw input images ✔ DONE

- Color space transforms and gradient thresholding to create binary images ✔ DONE

- Perspective transform (Birds-Eye View) to rectify binary images ✔ DONE

- Lane pixel detection using histogram-based sliding window search ✔ DONE

- Polynomial curve fitting to identify lane boundaries ✔ DONE

- Lane curvature and vehicle offset calculation ✔ DONE

- Inverse perspective transforms to overlay results on original frames ✔ DONE

## 1.4 Key Technologies Used

| Technology | Purpose | Version/Type |
| --- | --- | --- |
| Python | Primary programming language | Python 3.x |
| OpenCV (cv2) | Image processing and computer vision | 3.x / 4.x |
| NumPy | Numerical computation and array operations | Latest stable |
| Matplotlib | Visualization and image display | Latest stable |
| MoviePy | Video processing and output | Latest stable |
| Jupyter Notebook | Development and experimentation environment | Latest stable |
| Anaconda | Environment and package management | Python 3 version |

# 2. Technical Implementation

All planned algorithms were successfully implemented. The following table summarizes the implementation status of each technique:

## 2.1 Algorithm Implementation Status

| Algorithm / Technique | Description | Status |
| --- | --- | --- |
| Camera Calibration | Maps 3D real-world points to 2D image points using chessboard patterns | ✔ Implemented |
| Sobel Edge Detection | Gradient-based edge detection in X and Y directions | ✔ Implemented |
| HLS Color Space Transform | Separates hue, lightness, saturation to isolate yellow/white lanes | ✔ Implemented |
| Morphological Top-Hat | Isolates bright regions (lane lines) from surroundings | ✔ Implemented |
| Perspective Transform | Warps image to birds-eye view for accurate lane curvature measurement | ✔ Implemented |
| Sliding Window Search | Detects lane pixels column by column using histogram peaks | ✔ Implemented |
| Polynomial Fitting (2nd order) | Fits a curved line through detected lane pixels | ✔ Implemented |
| Radius of Curvature | Converts pixel-space polynomial to real-world measurement (meters) | ✔ Implemented |

## 2.2 Software & Tools

All required software tools were open-source and freely available. The project was fully implemented on standard academic hardware with no proprietary licenses required.

## 2.3 Dataset Used

The project used the publicly available Udacity self-driving car dataset, which provided dashcam video footage under various road and lighting conditions:

- Standard road footage (straight and curved lane scenarios) — tested successfully

- Challenge video with varying lighting, shadows, and pavement colors — tested with mitigation strategies applied

- Harder challenge video with sharp curves and rapid lighting changes — tested; limitations documented

## 2.4 Challenges Encountered & Resolutions

| Challenge | Impact | Mitigation Applied | Outcome |
| --- | --- | --- | --- |
| Shadows and lighting variation | High — affects edge/color thresholds | Top-Hat morphological operation + adaptive thresholding | ✔ Resolved |
| Sharp road curves | Medium — pipeline fails on tight bends | Reduced perspective transform region of interest | ✔ Resolved |
| Road crossings or lane merges | High — pipeline not designed for these | Scope limited to highway/freeway scenarios | Documented as limitation |
| Rain, snow, night conditions | High — not in current pipeline | Documented as future work | Noted for future extension |
| Processing speed for real-time use | Medium — single-frame pipeline is slow | Acceptable for academic demo | ✔ Acceptable for scope |

# 3. Operational Feasibility — Confirmed

The project was carried out by a two-member team with intermediate Python and basic computer vision knowledge. The modular Jupyter Notebook architecture supported iterative development and debugging throughout the semester.

## 3.1 Skills Applied

| Skill Area | Level Required | Status |
| --- | --- | --- |
| Python Programming | Intermediate | ✔ Applied successfully |
| Linear Algebra & Matrix Operations | Intermediate | ✔ Applied successfully |
| Computer Vision Fundamentals | Basic to Intermediate | ✔ Acquired through self-study |
| Image Processing Concepts | Basic to Intermediate | ✔ Applied successfully |
| OpenCV Library Usage | Basic | ✔ Applied successfully |
| Jupyter Notebook / Python IDE | Basic | ✔ Used throughout development |
| Git / GitHub Version Control | Basic | ✔ Repository maintained |

# 4. Financial / Economic Feasibility — Confirmed

## 4.1 Final Cost Summary

| Resource | Cost | Notes |
| --- | --- | --- |
| Python, OpenCV, NumPy, Matplotlib, MoviePy | Free | Open-source |
| Anaconda Distribution + Jupyter Notebook | Free | Free for academic use |
| Udacity Dataset + Caltech Lanes Dataset | Free | Publicly available on GitHub |
| GitHub Repository Hosting | Free | Public repository |
| Hardware (Personal Computer) | Already owned | Standard laptop sufficient |
| GPU Acceleration | Not required | Classical CV — no deep learning needed |
| TOTAL COST | PKR 0 / $0 | Fully open-source project |

## 4.2 Academic Value Generated

- Strong portfolio project demonstrating a real-world AI/ML application

- Deep understanding of computer vision — a highly demanded industry skill

- Practical hands-on experience with industry-standard tools (OpenCV, Python, Jupyter)

- Foundation for future research in autonomous vehicles or deep learning extensions

- Demonstrable video output suitable for interviews, presentations, and portfolios

# 5. Schedule / Timeline — Completed

## 5.1 Development Timeline — All Phases Completed

| Phase | Activities | Status |
| --- | --- | --- |
| Phase 1: Setup & Research | Environment setup, literature review, dataset download | ✔ Done |
| Phase 2: Camera Calibration | Chessboard calibration, undistortion pipeline | ✔ Done |
| Phase 3: Image Preprocessing | Color transforms, Sobel gradients, binary thresholding, Top-Hat | ✔ Done |
| Phase 4: Perspective Transform | Birds-eye view warp and inverse warp functions | ✔ Done |
| Phase 5: Lane Detection | Sliding window search, polynomial fitting, lane pixel ID | ✔ Done |
| Phase 6: Measurement & Overlay | Curvature radius, vehicle offset, overlay on frames | ✔ Done |
| Phase 7: Video Pipeline | Full pipeline on video, MoviePy integration | ✔ Done |
| Phase 8: Testing & Optimization | Challenge videos, threshold tuning, edge cases | ✔ Done |
| Phase 9: Documentation & Report | Final report, presentation, demo video | ✔ Done |
| Phase 10: Final Presentation | Presented to faculty, deliverables submitted | ✔ Done |

# 6. Overall Completion Summary

| Feasibility Dimension | Verdict | Confidence Level |
| --- | --- | --- |
| Technical Feasibility | COMPLETED | High |
| Operational Feasibility | COMPLETED | High |
| Financial / Economic Feasibility | COMPLETED (Zero Cost) | Very High |
| Schedule / Time Feasibility | COMPLETED (Within Semester) | High |

## 6.1 Final Conclusion

The Advanced Lane Detection project has been fully implemented and is hereby submitted as a completed university-level AI/ML project. All pipeline stages proposed in the original feasibility study were successfully built and tested. The project is technically sound, delivered at zero financial cost, completed within the semester timeline, and produces a visually compelling demonstrable video output.

## 6.2 Strengths Realized

- Zero financial cost — entirely open-source tools and datasets

- Strong real-world relevance — autonomous driving is a leading AI research area

- Visually compelling output — video demonstration clearly showcases results

- Modular architecture — implemented, tested, and debugged incrementally

- Rich existing literature and community resources leveraged throughout development

- Clear academic deliverables produced: code, report, and demo video

## 6.3 Future Enhancements Recommended

- Add a deep learning comparison module (e.g., CNN-based lane segmentation) to strengthen the AI/ML component

- Implement a performance metrics section (accuracy, F1-score on labeled frames)

# 7. Contribution Matrix

GitHub Repository: https://github.com/MazherShah/Lane-Detection

| Name | Contribution |
| --- | --- |
| Jahanzaib Ibrahim Khan | Project Overview, Technical Feasibility, Camera Calibration, Image Preprocessing implementation |
| Mazher Ali Shah | Operational Feasibility, Financial Feasibility, Time Feasibility, Perspective Transform, Lane Detection, Video Pipeline, Testing |

# 8. References

1. Ayanzadeh, A. (2017). Udacity Advanced Lane Detection of the Road. GitHub Repository. https://github.com/Ayanzadeh93/Udacity-Advance-Lane-detection-of-the-road

2. Udacity Self-Driving Car Engineer Nanodegree — Advanced Lane Finding Project. https://www.udacity.com/self-driving-car

3. Bradski, G. (2000). The OpenCV Library. Dr. Dobb's Journal of Software Tools.

4. Dickmanns, E. D., & Mysliwetz, B. D. (1992). Recursive 3-D road and relative ego-state recognition. IEEE Trans. Pattern Anal. Mach. Intell., 14(2), 199-213.

5. Saudi, T., Hijazi, J., & Sulaiman, J. (2008). Fast lane detection with randomized Hough transform. Proc. Symp. Inf. Technol., 4, 1-5.

6. Wang, J., Wu, Y., Liang, Z., & Xi, Y. (2010). Lane detection based on random Hough transform on region of interest. Proc. IEEE Conf. Inform. Autom., 1735-1740.

7. Caltech Lanes Dataset. http://www.mohamedaly.info/datasets/caltech-lanes

8. Comma.ai Public Driving Dataset. https://github.com/commaai/research